# Project Based Learning:

# SMART PARKING SYSTEM USING ATmega328

# Project Overview:

The Smart Parking System is an embedded systems project developed using the ATmega328P microcontroller. It helps manage parking spaces by displaying the number of available slots and controlling a gate using a servo motor. A 16×2 LCD shows the parking status and available spaces.

This project was designed and tested using SimulIDE. Since IR sensors were not available, push buttons were used to simulate vehicle entry and exit. The same program can be used with active-low IR sensors in a real hardware setup.

The project demonstrates basic embedded programming concepts such as GPIO interfacing, LCD interfacing, servo motor control, and digital input handling using Embedded C.

# Objectives:

Design a smart parking system.

Display available parking slots.

Control gate using servo motor.

Learn AVR programming.

Test the design using SimulIDE.

- Components used

| - COMPONENTS | - PURPOSE |
| --- | --- |
| ATmega328P. | Controller. |
| - LCD 16×2 | Display. |
| Servo Motor. | Gate control. |
| SimulIDE. | Simulation. |
| Push Buttons | Simulate IR sensors. |

- FLOW CHART

START

│

▼

Initialize LCD & Servo

│

▼

Set Available Slots = 2

│

▼

Display Status

│

▼

Is ENTRY Button Pressed?

┌───────┴────────┐

NO               YES

│                 │

│        Available Slots > 0?

│          ┌──────┴───────┐

│         NO                                     YES

│         │                                         │

│         ▼                                        ▼

│   Display "Parking                 Open Gate

│   Full / No Entry"                        │

│                                                    ▼

│                                            Decrease Slot

│                                                      │

│                                                     ▼

│                                                                Close Gate

│                                                                           │

└──────────────┬───────────┘

▼

Is EXIT Button Pressed?

┌─────────┴─────────┐

NO                                                YES

│                                                     │

│                                                     ▼

│                                                Open Gate

│                                                    │

│                                                    ▼

│                                           Increase Slot

│                                                  │

│                                                  ▼

│                                             Close Gate

│                                                        │

└───────────┬────────┘

▼

Update LCD

│

▼

Repeat Loop

System Initialization

LCD_Init();

Servo_Init();

Servo_CloseGate();

availableSlots = 2;

DisplayStatus();

Purpose: Initializes the LCD, servo motor, sets the initial parking slots, and displays the initial status.

Display Parking Status

void DisplayStatus(void)

{

LCD_Clear();

LCD_SetCursor(0,0);

LCD_String("Available:");

LCD_SetCursor(0,11);

LCD_Char(availableSlots + '0');

if(availableSlots == 0)

LCD_String("PARKING FULL");

else

LCD_String("WELCOME");

}

Explanation:

- Displays the number of available parking slots.

- Shows WELCOME when spaces are available.

- Shows PARKING FULL when no spaces are left.

Applications

- Shopping malls

- Hospitals

- Apartments

- Office buildings